SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE NAME: Computer Networks                                       CSA07 COURSE CODE: CSA07

COURSE OUTCOME COVERED

CO1: Apply the functions of OSI layer in enabling reliable data transmission across networks. (BL2, BL3)

Assessment Tool Weightage: 40%

Description about the assessment tool:

Short-answer questions effectively measure conceptual understanding, logical reasoning, and the ability to apply theoretical knowledge without requiring lengthy descriptive responses. This assessment method is also efficient for evaluating multiple OSI-related concepts within a limited time while ensuring objective and consistent evaluation.

Short Answer Test

1.A company needs to transfer a 150 MB design file from a client computer to a remote server over a network with a bandwidth of 50 Mbps. The Transport Layer divides the file into 1500-byte segments, while the Data Link Layer encapsulates each segment into frames by adding 40 bytes of header and trailer. Calculate the total number of segments and frames transmitted, the total transmission overhead introduced by the Data Link Layer, and estimate the total transmission time (ignoring propagation delay). Explain how the Transport and Data Link layers ensure reliable communication.

Solution:

### Given:

- File size = 150 MB

- Bandwidth = 50 Mbps

- Segment size = 1500 bytes

- Data Link overhead/frame = 40 bytes

### Step 1: Convert file size to bytes

150 MB = 150 × 1024 × 1024

= 157,286,400 bytes

### Step 2: Number of segments

157,286,4001500=104,858 segments (approx.)\frac{157,286,400}{1500}=104,858 \text{ segments (approx.)}1500157,286,400​=104,858 segments (approx.)

So,

Number of segments = 104,858

Each segment becomes one frame.

Number of frames = 104,858

### Step 3: Total Data Link overhead

104,858×40=4,194,320 bytes104,858 \times 40=4,194,320 \text{ bytes}104,858×40=4,194,320 bytes

≈ 4 MB

### Step 4: Total transmission size

157,286,400+4,194,320=161,480,720 bytes157,286,400+4,194,320=161,480,720 \text{ bytes}157,286,400+4,194,320=161,480,720 bytes

Convert to bits:

161,480,720×8=1,291,845,760 bits161,480,720\times8=1,291,845,760 \text{ bits}161,480,720×8=1,291,845,760 bits

Transmission time

1,291,845,76050×106=25.84 seconds\frac{1,291,845,760}{50\times10^6}=25.84\text{ seconds}50×1061,291,845,760​=25.84 seconds

### Answer

- Number of segments = 104,858

- Number of frames = 104,858

- Total overhead = 4,194,320 bytes

- Transmission time ≈ 25.84 s

### Explanation

- Transport Layer: Segments data, provides sequencing, acknowledgements, and error recovery.

- Data Link Layer: Frames data, performs error detection (CRC), and ensures reliable node-to-node delivery.

2. A Transport Layer protocol uses a sliding window size of 6 frames. Each frame carries 1024 bytes of user data. If the sender transmits 48 frames, calculate the number of transmission windows required. If one frame in every window is lost and must be retransmitted, determine the total number of retransmissions. Explain how flow control improves communication efficiency.

Solution:

### Given

- Window size = 6 frames

- Total frames = 48

### Step 1: Number of windows

486=8\frac{48}{6}=8648​=8

### Step 2: Retransmissions

One frame lost in each window

Total retransmissions

= 8

### Answer

- Transmission windows = 8

- Retransmissions = 8 frames

### Explanation

Flow control prevents the sender from overwhelming the receiver, reducing congestion and improving network efficiency.

3. A 12,000-byte IP packet is transmitted through a router where the Maximum Transmission Unit (MTU) is 1500 bytes. Assuming each fragment requires a 20-byte IP header, calculate the total number of fragments created and the total header overhead introduced. Explain how the Network Layer ensures successful delivery of fragmented packets to the destination.

Solution:

### Given

- Packet size = 12,000 bytes

- MTU = 1500 bytes

- IP header = 20 bytes

### Step 1: Payload per fragment

1500−20=1480 bytes1500-20=1480\text{ bytes}1500−20=1480 bytes

### Step 2: Number of fragments

120001480=8.1\frac{12000}{1480}=8.1148012000​=8.1

Round up

= 9 fragments

### Step 3: Header overhead

9×20=180 bytes9\times20=180\text{ bytes}9×20=180 bytes

### Answer

- Fragments = 9

- Header overhead = 180 bytes

### Explanation

The Network Layer fragments packets when required and reassembles them at the destination using fragment offsets and identification numbers.

4. A Transport Layer protocol uses a sliding window size of 6 frames. Each frame carries 1024 bytes of user data. If the sender transmits 48 frames, calculate the number of transmission windows required. If one frame in every window is lost and must be retransmitted, determine the total number of retransmissions. Explain how flow control improves communication efficiency.

Solution:

### Given

- Window size = 6 frames

- Total frames = 48

### Step 1: Number of windows

486=8\frac{48}{6}=8648​=8

### Step 2: Retransmissions

One frame lost in each window

Total retransmissions

= 8

### Answer

- Transmission windows = 8

- Retransmissions = 8 frames

### Explanation

Flow control prevents the sender from overwhelming the receiver, reducing congestion and improving network efficiency.

5. A multimedia application transfers a 600 MB video file. The Session Layer inserts a synchronization checkpoint after every 60 MB of transmitted data. During transmission, a failure occurs after 390 MB has been transferred. Determine the number of checkpoints created before failure and calculate the amount of data that must be retransmitted after recovery. Explain the importance of synchronization in the Session Layer.

Solution:

### Given

- File size = 600 MB

- Checkpoint every 60 MB

- Failure after 390 MB

### Step 1: Checkpoints before failure

39060=6.5\frac{390}{60}=6.560390​=6.5

Completed checkpoints

= 6

(Checkpoints at 60,120,180,240,300,360 MB)

### Step 2: Data retransmitted

390−360=30 MB390-360=30\text{ MB}390−360=30 MB

### Answer

- Checkpoints created = 6

- Retransmitted data = 30 MB

### Explanation

Synchronization allows communication to restart from the last checkpoint instead of retransmitting the entire file.

6. Before transmission, the Presentation Layer compresses a 900 MB file by 40%. The compressed data is encrypted, adding 5% overhead to the compressed size. Calculate the compressed file size, the encrypted file size, and the total percentage reduction compared to the original file. Explain how the Presentation Layer improves transmission efficiency and security.

Solution:

### Given

- Original file = 900 MB

- Compression = 40%

- Encryption overhead = 5%

### Step 1: Compressed size

40% removed

Remaining

900×0.60=540 MB900\times0.60=540\text{ MB}900×0.60=540 MB

### Step 2: Encryption overhead

540×1.05=567 MB540\times1.05=567\text{ MB}540×1.05=567 MB

### Step 3: Percentage reduction

900−567900×100=37%\frac{900-567}{900}\times100=37\%900900−567​×100=37%

### Answer

- Compressed size = 540 MB

- Encrypted size = 567 MB

- Total reduction = 37%

### Explanation

The Presentation Layer compresses data for faster transmission and encrypts it for confidentiality.

7.An FTP application transfers 3 GB of data over a network with an effective throughput of 120 Mbps. The Application Layer divides the data into requests of 3 MB each. Calculate the total number of requests generated and estimate the total transmission time in seconds. Discuss how the Application Layer provides reliable services to users.

Solution:

### Given

- Data = 3 GB

- Throughput = 120 Mbps

- Request size = 3 MB

### Step 1: Convert to MB

3GB=3072MB3GB=3072MB3GB=3072MB

### Step 2: Requests

30723=1024\frac{3072}{3}=102433072​=1024

### Step 3: Transmission time

Convert to bits

3072MB×8=24,576Mb3072MB\times8=24,576Mb3072MB×8=24,576Mb

Time

24576120=204.8 s\frac{24576}{120}=204.8\text{ s}12024576​=204.8 s

### Answer

- Requests = 1024

- Transmission time = 204.8 s

### Explanation

The Application Layer provides services such as FTP, authentication, file management, and reliable user communication.

8.A packet experiences the following processing delays at different OSI layers: Application Layer = 4 ms, Transport Layer = 3 ms, Network Layer = 5 ms, Data Link Layer = 2 ms, and Physical Layer = 1 ms. The propagation delay is 18 ms, and the transmission delay is 12 ms. Calculate the total end-to-end delay experienced by the packet and explain how each layer contributes to the communication process.

Solution:

### Given

Application = 4 ms

Transport = 3 ms

Network = 5 ms

Data Link = 2 ms

Physical = 1 ms

Propagation = 18 ms

Transmission = 12 ms

### Total delay

4+3+5+2+1+18+12=45ms4+3+5+2+1+18+12=45ms4+3+5+2+1+18+12=45ms

### Answer

Total end-to-end delay = 45 ms

### Explanation

- Application Layer processes user requests.

- Transport Layer manages reliable delivery.

- Network Layer routes packets.

- Data Link Layer frames data.

- Physical Layer transmits bits.

- Propagation delay is the travel time through the medium.

- Transmission delay is the time to place bits onto the link.

9.A network transfers 80 MB of useful data using frames of 1600 bytes, where each frame contains 80 bytes of protocol overhead added by different OSI layers. Calculate the total number of frames required, the total overhead transmitted, and the transmission efficiency as a percentage. Explain how protocol overhead affects network performance.

Solution:

### Given

- Useful data = 80 MB

- Frame size = 1600 bytes

- Overhead = 80 bytes

### Step 1: Useful payload/frame

1600−80=1520 bytes1600-80=1520\text{ bytes}1600−80=1520 bytes

### Step 2: Useful data

80MB=83,886,080 bytes80MB=83,886,080\text{ bytes}80MB=83,886,080 bytes

### Step 3: Number of frames

83,886,0801520=55,188.2\frac{83,886,080}{1520}=55,188.2152083,886,080​=55,188.2

Round up

= 55,189 frames

### Step 4: Total overhead

55,189×80=4,415,120 bytes55,189\times80=4,415,120\text{ bytes}55,189×80=4,415,120 bytes

≈ 4.21 MB

### Step 5: Transmission efficiency

15201600×100=95%\frac{1520}{1600}\times100=95\%16001520​×100=95%

### Answer

- Frames = 55,189

- Total overhead = 4,415,120 bytes

- Efficiency = 95%

### Explanation

Protocol overhead reduces bandwidth available for useful data, slightly lowering transmission efficiency

10.A hospital transfers a patient's medical record of 240 MB through an IoT-enabled healthcare network. The Presentation Layer compresses the data by 30%, the Transport Layer divides the compressed data into 1400-byte segments, and the Data Link Layer adds 60 bytes of overhead to each frame. Calculate the compressed file size, the total number of segments transmitted, the total protocol overhead introduced, and the final amount of data transmitted across the Physical Layer. Explain how multiple OSI layers work together to ensure secure and reliable communication.

Solution:

### Given

- Original file = 240 MB

- Compression = 30%

- Segment size = 1400 bytes

- Frame overhead = 60 bytes

### Step 1: Compressed size

240×0.70=168MB240\times0.70=168MB240×0.70=168MB

Convert to bytes

168×1024×1024=176,160,768 bytes168\times1024\times1024=176,160,768\text{ bytes}168×1024×1024=176,160,768 bytes

### Step 2: Segments

176,160,7681400=125,829.12\frac{176,160,768}{1400}=125,829.121400176,160,768​=125,829.12

Round up

= 125,830 segments

### Step 3: Protocol overhead

125,830×60=7,549,800 bytes125,830\times60=7,549,800\text{ bytes}125,830×60=7,549,800 bytes

≈ 7.2 MB

### Step 4: Final transmitted data

176,160,768+7,549,800=183,710,568 bytes176,160,768+7,549,800=183,710,568\text{ bytes}176,160,768+7,549,800=183,710,568 bytes

≈ 175.2 MB

### Answer

- Compressed file size = 168 MB

- Segments = 125,830

- Protocol overhead = 7,549,800 bytes

- Final transmitted data = 183,710,568 bytes (≈175.2 MB)

### Explanation

- Presentation Layer: Compresses and secures the medical record.

- Transport Layer: Divides data into reliable segments.

- Data Link Layer: Adds frame headers/trailers for error detection.

- Physical Layer: Transmits the final bit stream across the network.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understanding of Concepts | 25% | Demonstrates a thorough, accurate understanding of concepts | Good understanding with minor gaps | Basic understanding with some misconceptions | Poor understanding; major misconceptions |
| Explanation | 25% | Detailed, well-explained answers with relevant examples | Clear explanation with adequate details | Limited explanation; lacks depth | Poor or unclear explanation |
| Application | 20% | Effectively applies concepts with strong analytical ability | Applies concepts with minor errors | Limited application with weak analysis | No application or incorrect analysis |
| Organization | 15% | Well-structured answers with logical flow and coherence | Organized with minor issues in flow | Basic structure, lacks clarity | Poor organization, incoherent answers |
| Accuracy | 10% | Completely accurate and covers all required points | Mostly accurate with minor omissions | Partially correct with missing points | Incorrect answers with major gaps |
| Clarity | 5% | Clear, neat, professional presentation | Understandable with minor issues | Limited clarity, readability issues | Poorly written, difficult to understand |